A RISC-V-Based Server Baseboard Management Controller (BMC) SoC with Hardware Fault Detection, Automated Recovery, and Live VGA Status Monitoring

# 1. Problem Statement

Server hardware relies on a small, independent controller — a Baseboard Management Controller (BMC), commercially implemented as Dell iDRAC, HP iLO, or the open IPMI standard — to monitor system health and recover a frozen or unresponsive machine without requiring a human to be physically present. This out-of-band management capability is foundational to modern datacenter reliability, yet its internal hardware/software design is proprietary, closed-source, and essentially invisible to students.

This project addresses that gap by designing a System-on-Chip, built around a provided RISC-V RTL processor core, that implements the core functions of a BMC: continuous heartbeat-based health monitoring, automated fault recovery via a controlled reset sequence, and — critically — a recovery policy that recognizes when repeated automatic restarts are failing to resolve the underlying fault and escalates to a locked-out, human-attention-required state rather than looping indefinitely. All system state is surfaced through both a UART-based remote management console and a live, real-time VGA status dashboard. The objective is to demonstrate, through RTL design and simulation-based verification, that a small set of tightly-coupled memory-mapped IP blocks — heartbeat detection, reset sequencing, and stateful recovery policy — can deliver a complete detect-recover-escalate reliability pipeline representative of real server management hardware, implemented and verified in an academic setting.

# 2. Proposed SoC Architecture

The SoC is organized around the provided RISC-V processor core as the central control and policy-decision unit, connected via a shared memory-mapped AXI/Wishbone bus interconnect to instruction memory, data memory, and a set of peripheral and monitoring IP blocks. The processor's role is limited to reading detection status, applying recovery policy, and servicing the management console; continuous heartbeat timing and reset-pulse generation are performed by dedicated hardware, independent of whether the core — or the system it is monitoring — is behaving correctly. The system follows this operational pipeline:

Detection — the Heartbeat Monitor IP continuously watches a periodic "alive" pulse from a simulated main system (driven via GPIO), using an internal hardware counter to measure elapsed time since the last pulse. If the elapsed time exceeds a processor-configured threshold, the IP latches an unresponsive flag entirely in hardware, so detection does not depend on software that could itself be affected by the fault.

Recovery — upon an unresponsive flag (or a manual override command), the Power/Reset Sequencer IP drives a precisely-timed reset pulse — asserted for a configurable hold duration and then released — to the simulated main system via GPIO, mirroring how real server management hardware performs a controlled power-cycle rather than a bare signal toggle.

Policy & Logging — the Recovery Policy / Event Log IP records every recovery event with a timestamp (from the Timer) into a circular log buffer and tracks how many recoveries have occurred within a configurable rolling window. If that count exceeds a processor-configured threshold, the IP sets a sticky lockout flag that blocks further automatic recovery — the design's central reliability argument: a system that keeps failing should stop being blindly restarted and should instead be flagged for human attention.

Monitoring & Control — the processor reports live status, event history, and accepts commands (status, history, force reset) over the mandatory UART peripheral, and separately drives the VGA Controller to render a real-time visual dashboard — current status (ONLINE / UNRESPONSIVE / LOCKED OUT), time since last heartbeat, restart count versus threshold, and a scrolling event log — viewable live during simulation.

This architecture keeps the RISC-V core off the time-critical detection and reset-timing paths — it only ever reads status and writes policy/configuration state, never itself measures the heartbeat interval or holds the reset line — mirroring how real BMC hardware is designed to remain trustworthy even when the system it manages is not. The complete memory map, Wishbone register interfaces, and per-block timing diagrams for each custom IP will be specified in the accompanying Architecture Document.

# 3. Selected IPs

Mandatory (required by the SoC architecture; integrated from proven open-source cores):

RISC-V Processor (provided) — central control unit; runs the BMC firmware and applies recovery policy.

Instruction Memory and Data Memory — program storage and event-log/configuration staging.

Bus Interconnect (AXI/Wishbone Bus) — memory-mapped fabric connecting the core to all peripherals.

UART — remote management console for status queries and manual override commands.

Timer — provides timestamps for event logging and elapsed-time measurement.

GPIO — carries the simulated heartbeat signal in and the reset/power line out.

Additional IPs (three required, selected below):

Heartbeat Monitor — a memory-mapped hardware block that measures elapsed cycles since the last heartbeat pulse and latches an unresponsive flag in hardware when a processor-configured threshold is exceeded, ensuring detection remains independent of software that could itself be affected by the fault it is meant to catch.

Power/Reset Sequencer — a memory-mapped block that drives a controlled, precisely-timed reset pulse to the simulated main system on trigger (automatic or manual), holding the reset line for a configurable duration before release — the actual recovery mechanism of the system.

Recovery Policy / Event Log — a memory-mapped block combining a timestamped circular event log with rolling-window recovery counting; sets a sticky lockout flag when repeated recoveries exceed a configurable threshold, giving the system the ability to recognize a crash-loop and stop auto-restarting rather than retrying indefinitely.

VGA Controller (optional fourth IP) — a display peripheral, adapted from a base VGA sync/pixel-logic core, that renders a live status dashboard — current state, elapsed heartbeat time, restart count, and a scrolling event log — read directly from the other custom IPs' memory-mapped state, giving the system a real-time visual monitoring output in addition to the UART console.

Together, these blocks satisfy the required scope of the assignment — a RISC-V-centered SoC with a complete memory map, at least three integrated IPs, an application (the health-monitoring, recovery-policy, and console/dashboard firmware) running on the processor, and functionality verifiable through RTL simulation.