BMC SoC — Full Project Specification, Manual & Architecture Document

RISC-V-Based Server Baseboard Management Controller with Hardware Fault Detection, Automated Recovery, and Live VGA Status Monitoring

# 1. Introduction

## 1.1 Background

Real server hardware relies on a small, independent controller — a Baseboard Management Controller (BMC), commercially implemented as Dell iDRAC, HP iLO, or the open IPMI standard — to monitor system health and recover a frozen or unresponsive machine without requiring a human to be physically present. This "out-of-band" management capability is foundational to modern datacenter reliability. Its internal hardware/software design is, however, proprietary and effectively invisible to students — nobody can open a commercial BMC and study how it decides when a system has hung, how it recovers it, or how it decides when to stop trying.

## 1.2 Project Objective

This project implements a simplified BMC as a RISC-V System-on-Chip, built around the provided VeeR EL2 processor core (RV32IMC). The system performs three core functions, each implemented as dedicated hardware:

1. Detection — continuously monitor a periodic heartbeat signal from a simulated "main system" and declare it unresponsive if the signal stops, using a hardware-timed mechanism entirely independent of software.

2. Recovery — automatically trigger a controlled, precisely-timed reset sequence to attempt to bring the unresponsive system back online.

3. Policy & Escalation — track how frequently recoveries occur; if the monitored system is crash-looping (repeated failures within a short time window), lock out further automatic recovery and escalate to a state requiring human intervention rather than restarting indefinitely.

All system state is exposed through a UART-based remote management console and a live VGA status dashboard, mirroring how real BMC hardware is both monitored and operated by a system administrator.

## 1.3 Core Design Principle

The processor is strictly a configuration and policy layer. It never itself measures elapsed time for fault detection, never holds a reset line for a timed duration, and is never the thing verifying its own health. All time-critical, correctness-critical behavior lives in dedicated hardware blocks, so the system remains trustworthy even if the software running on it misbehaves — this is the same design principle that makes real watchdog and management hardware meaningful: the detector cannot be vulnerable to the same class of failure it exists to catch.

## 1.4 Scope Summary

| Category | Components |
| --- | --- |
| Provided / integrated, not designed | VeeR EL2 core, instruction/data memory, AXI4 interconnect, AXI-to-Wishbone bridge, UART, Timer, GPIO |
| Custom-designed (primary project contribution) | Heartbeat Monitor, Power/Reset Sequencer, Recovery Policy/Event Log |
| Custom-designed (optional 4th IP) | VGA Controller (adapted from a base sync/pixel-logic core) |
| Explicitly out of scope | The "main system" being monitored — represented entirely as testbench-driven stimulus, not real hardware |

## 1.5 Document Purpose

This document is the single consolidated reference for the project: complete signal-level interfaces, control and data path behavior, error handling, interrupt architecture, full register specifications, and the firmware/C application design. It supersedes the separate architecture, specification, and presentation-prep documents produced earlier in the design process, consolidating them into one authoritative manual.

# 2. Block Diagram

Already produced separately — see `BMC_Architecture_Block_Diagrams.drawio`. Reproduced here as a reference only; refer to the drawio source for the maintained, editable version.

Top-level composition: VeeR EL2 Core → AXI4 Interconnect → AXI-to-Wishbone Bridge → Wishbone peripheral bus, with UART, GPIO, Timer (mandatory) and Heartbeat Monitor, Power/Reset Sequencer, Recovery Policy/Log, VGA Controller (custom) all as Wishbone slaves. heartbeat_in and reset_out are direct top-level chip I/O, wired point-to-point to the Heartbeat Monitor and Power/Reset Sequencer respectively, bypassing GPIO and the Wishbone bus entirely.

# 3. Custom IP — Detailed Functionality, Algorithms & Behavior

This section describes, for each custom-designed IP, its full functional behavior, the governing algorithm in step-by-step form, its behavior across normal and edge-case scenarios, and its finite state machine. Diagram placeholders are marked explicitly; insert the corresponding figures from the accompanying .docx/.md diagram set at each marked location.

## 3.1 Heartbeat Monitor — Detailed Functionality

### 3.1.1 Functional Description

The Heartbeat Monitor is a purely reactive hardware observer. It has exactly one job: continuously measure how much time has elapsed since the last valid pulse on heartbeat_in, and raise a flag the moment that elapsed time exceeds a configurable threshold. It has no awareness of *why* the pulse stopped — only *that* it stopped. This narrow, single-purpose behavior is deliberate: a detector with a small, easily-verified job is far easier to trust than one with complex logic.

The IP operates continuously and autonomously once enabled — it requires no ongoing firmware interaction to perform detection. Firmware's only responsibilities are: (1) configure the threshold once (or update it if policy changes), (2) periodically read the status, and (3) explicitly acknowledge/clear a detected event.

[DIAGRAM PLACEHOLDER — Figure 3.1.1: Heartbeat Monitor block-level internal diagram, showing the counter, edge detector, comparator, and register file. Insert from diagram set.]

### 3.1.2 Governing Algorithm

ALGORITHM: Heartbeat Monitor (executed every clock cycle in hardware) INPUT: heartbeat_in (1-bit), HB_THRESHOLD (32-bit, firmware-configured) OUTPUT: HB_STATUS.unresponsive (1-bit), HB_ELAPSED (32-bit) 1. IF NOT enabled: counter <- 0 state <- DISABLED RETURN 2. sample <- heartbeat_in rising_edge <- sample AND (NOT previous_sample) previous_sample <- sample 3. IF state == UNRESPONSIVE: counter <- counter + 1 // keep growing for HB_ELAPSED visibility // remain latched regardless of rising_edge - see 3.1.4 RETURN 4. IF rising_edge: counter <- 0 // valid pulse arrived in time - reset state <- COUNTING RETURN 5. IF counter >= HB_THRESHOLD: unresponsive_flag <- 1 // latch - hardware decision, not firmware state <- UNRESPONSIVE ELSE: counter <- counter + 1 6. HB_ELAPSED <- counter // always live-readable HB_STATUS.bit0 <- unresponsive_flag HB_STATUS.bit1 <- heartbeat_in (live level, informational)

### 3.1.3 Finite State Machine

[DIAGRAM PLACEHOLDER — Figure 3.1.3: Heartbeat Monitor FSM diagram (DISABLED → IDLE → COUNTING → UNRESPONSIVE, with the explicit-clear transition back to COUNTING). Insert from diagram set.]

| From State | Condition | To State | Action |
| --- | --- | --- | --- |
| DISABLED | enable == 1 | IDLE | counter held at 0 |
| IDLE | any cycle | COUNTING | begin counting |
| COUNTING | rising edge on heartbeat_in | COUNTING | counter resets to 0 |
| COUNTING | counter >= HB_THRESHOLD | UNRESPONSIVE | latch flag, keep counting for HB_ELAPSED |
| UNRESPONSIVE | firmware writes HB_CTRL.clear=1 | COUNTING | flag cleared, counter resets to 0 |
| UNRESPONSIVE | (heartbeat resumes, no clear write) | UNRESPONSIVE | no transition — flag remains latched (see 3.1.4) |
| any | enable == 0 | DISABLED | counter reset, monitoring halted |

### 3.1.4 Behavioral Notes — Latching Discipline

The single most important behavioral property of this IP: the unresponsive flag never self-clears on a resumed heartbeat. Only an explicit firmware write clears it. This converts the flag into a handshake rather than a passive status bit, guaranteeing firmware cannot miss a fault event due to unlucky timing (see Section 7.1.3 for the failure mode this prevents). This is a deliberate behavioral choice, verified directly by unit test (Section under Verification Plan, Test 3).

### 3.1.5 Behavior Across Scenarios

| Scenario | Behavior |
| --- | --- |
| Normal operation, regular pulses well under threshold | Counter continuously resets; flag never sets; HB_ELAPSED stays low |
| Pulse stream stops entirely | Counter grows unbounded (up to saturation, Section 7.1.4) until threshold crossed, then flag latches |
| Pulse resumes after flag has latched | Flag remains latched (3.1.4); HB_ELAPSED keeps growing until explicit clear |
| Firmware clears flag, pulses still absent | State returns to COUNTING, counter restarts from 0, will re-latch again once threshold is re-crossed |
| Monitor disabled mid-count | Counter resets to 0 immediately; no flag latched even if it was close to threshold |

## 3.2 Power/Reset Sequencer — Detailed Functionality

### 3.2.1 Functional Description

The Power/Reset Sequencer's job is to convert a single-cycle software trigger into a correctly-shaped, precisely-timed physical reset pulse. This separation matters: firmware only ever expresses *intent* ("reset now"); hardware is solely responsible for *how* that intent becomes a real, minimum-duration electrical pulse — timing that would be unreliable if left to firmware-driven bit-banging.

[DIAGRAM PLACEHOLDER — Figure 3.2.1: Power/Reset Sequencer block-level internal diagram, showing the countdown register, trigger latch, and output driver. Insert from diagram set.]

### 3.2.2 Governing Algorithm

ALGORITHM: Power/Reset Sequencer (executed every clock cycle in hardware) INPUT: trigger (pulsed, from RST_CTRL write), RST_HOLD_CYCLES (32-bit) OUTPUT: reset_out (1-bit), RST_STATUS (in_progress, complete) 1. IF state == IDLE: reset_out <- 1 // inactive (active-low) IF trigger == 1: countdown <- RST_HOLD_CYCLES state <- ASSERT RETURN 2. IF state == ASSERT: reset_out <- 0 // pulse asserted in_progress <- 1 IF countdown == 0: state <- DEASSERT ELSE: countdown <- countdown - 1 RETURN 3. IF state == DEASSERT: reset_out <- 1 // release state <- DONE RETURN 4. IF state == DONE: in_progress <- 0 complete <- 1 // sticky until next trigger state <- IDLE RETURN // Note: trigger received while state != IDLE is explicitly ignored (see 3.2.4)

### 3.2.3 Finite State Machine

[DIAGRAM PLACEHOLDER — Figure 3.2.3: Power/Reset Sequencer FSM diagram (IDLE → ASSERT → DEASSERT → DONE → IDLE), annotated with the ignored-retrigger self-loop on ASSERT/DEASSERT/DONE. Insert from diagram set.]

| From State | Condition | To State | Action |
| --- | --- | --- | --- |
| IDLE | trigger == 1 | ASSERT | load countdown, drive reset_out low |
| ASSERT | countdown reaches 0 | DEASSERT | — |
| ASSERT | trigger == 1 (re-trigger) | ASSERT | ignored, no re-arm (3.2.4) |
| DEASSERT | (immediate) | DONE | drive reset_out high |
| DONE | (immediate) | IDLE | set complete flag, clear in_progress |

### 3.2.4 Behavioral Notes — Re-Trigger Handling

A trigger received while a sequence is already in progress (ASSERT, DEASSERT, or DONE) is explicitly ignored rather than restarting or queuing. This guarantees every reset pulse that begins is completed at its full configured width — a partial or re-timed pulse could fail to reliably reset the monitored system. Firmware wishing to guarantee a fresh, independent pulse should poll RST_STATUS.complete before issuing a second trigger.

### 3.2.5 Behavior Across Scenarios

| Scenario | Behavior |
| --- | --- |
| Single trigger, system idle | Full-width pulse produced exactly as configured |
| Trigger while a pulse is already asserting | Ignored; current pulse completes unaffected |
| RST_HOLD_CYCLES changed mid-sequence | New value only takes effect on the *next* trigger — in-progress countdown uses the value latched at trigger time |
| Automatic and manual trigger requested nearly simultaneously | Serialized naturally by the single-master Wishbone bus (Section 8, master/slave discussion); second write simply arrives after the first completes its bus transaction |

## 3.3 Recovery Policy / Event Log — Detailed Functionality

### 3.3.1 Functional Description

This IP is the "judgment" layer of the system — the component that distinguishes this design from a bare watchdog timer. It has two intertwined responsibilities: maintaining a durable, timestamped record of every recovery event (the Event Log), and evaluating whether the *rate* of recent recoveries indicates a crash-loop, in which case it autonomously blocks further automatic recovery (the Policy). Both responsibilities are implemented in hardware so the decision to lock out is as trustworthy and tamper-resistant as the detection mechanism itself.

[DIAGRAM PLACEHOLDER — Figure 3.3.1: Recovery Policy / Event Log block-level internal diagram, showing the circular log buffer, write pointer, window counter, and lockout comparator. Insert from diagram set.]

### 3.3.2 Governing Algorithm — Event Recording

ALGORITHM: Record Event (triggered on POL_CTRL.record_event write) INPUT: POL_EVENT_TS (32-bit, firmware-staged timestamp) OUTPUT: log_buffer updated, log_count updated, window_recovery_count updated 1. log_buffer[log_wr_ptr] <- POL_EVENT_TS 2. log_wr_ptr <- (log_wr_ptr + 1) MOD LOG_DEPTH // circular wrap 3. IF log_count < LOG_DEPTH: log_count <- log_count + 1 // saturates, does not wrap 4. window_recovery_count <- window_recovery_count + 1 5. IF window_recovery_count > POL_THRESHOLD: lockout_flag <- 1 // sticky

### 3.3.3 Governing Algorithm — Rolling Window (Fixed-Reset Approximation)

ALGORITHM: Window Management (executed every clock cycle in hardware) INPUT: POL_WINDOW (32-bit) OUTPUT: window_counter, window_recovery_count (reset on window boundary) 1. window_counter <- window_counter + 1 2. IF window_counter >= POL_WINDOW: window_counter <- 0 window_recovery_count <- 0 // NOTE: lockout_flag is NOT reset here (3.3.5)

### 3.3.4 Finite State / Data Flow Diagram

[DIAGRAM PLACEHOLDER — Figure 3.3.4: Recovery Policy timing diagram showing window boundaries, recovery events landing within/across windows, and the point at which lockout_flag latches. Insert from diagram set.]

### 3.3.5 Behavioral Notes — Why Lockout Outlives the Window

lockout_flag is intentionally not cleared by a window reset (step 2 of 3.3.3 explicitly excludes it). Once the system has been judged to be crash-looping, only a deliberate human decision (POL_CTRL.clear_lockout) resumes automatic recovery — a subsequent quiet window does not by itself imply the underlying fault is fixed. This is the single most important policy decision in the whole project and should be highlighted in any presentation or defense.

### 3.3.6 Behavior Across Scenarios

| Scenario | Behavior |
| --- | --- |
| Single recovery, well within threshold | Logged; window_recovery_count increments; no lockout |
| Recoveries exceed threshold within one window | Lockout latches on the event that crosses the threshold |
| Recoveries exceed threshold, then window resets | window_recovery_count resets to 0, but lockout_flag remains set (3.3.5) |
| More than 16 events logged since last reset | Circular buffer overwrites oldest entries; log_count saturates at 16 (see also Section 7.3.1) |
| Admin issues clear lockout | lockout_flag clears immediately; automatic recovery resumes on the next detected fault |

## 3.4 VGA Controller — Detailed Functionality (Optional 4th IP)

### 3.4.1 Functional Description

The VGA Controller is the only custom IP that is purely observational — it produces no control-path decisions and does not participate in the detect/recover/escalate logic at all. Its sole job is to continuously render whatever status content firmware last wrote into its buffer, as a live video signal, completely decoupled from the timing of the rest of the system.

[DIAGRAM PLACEHOLDER — Figure 3.4.1: VGA Controller block diagram, showing the pixel clock divider, scan counters, dashboard buffer, and sync generator. Insert from diagram set.]

### 3.4.2 Governing Algorithm

ALGORITHM: VGA Pixel Generation (executed every pixel clock cycle) 1. h_count <- h_count + 1 2. IF h_count == H_TOTAL: h_count <- 0 v_count <- v_count + 1 IF v_count == V_TOTAL: v_count <- 0 3. hsync <- (h_count within horizontal sync window) ? 0 : 1 vsync <- (v_count within vertical sync window) ? 0 : 1 active_video <- (h_count, v_count) within visible display area 4. IF active_video: content <- LOOKUP(dashboard_buffer, h_count, v_count) rgb <- content ELSE: rgb <- BLANK

### 3.4.3 Behavioral Notes

Firmware writes to dashboard_buffer are asynchronous to pixel generation — the controller always renders the most recently written content, with no explicit synchronization required for this application (a torn frame during a buffer update is a cosmetically acceptable, non-fatal condition given the buffer updates roughly once per main-loop iteration, far slower than the pixel/frame rate).

### 3.4.4 Behavior Across Scenarios

| Scenario | Behavior |
| --- | --- |
| Normal operation | Dashboard updates once per main-loop iteration; smoothly reflects current status |
| Firmware busy, buffer not updated for a while | Display shows stale (but not corrupted) content until next write; see Error 7.4.1 |
| VGA_CTRL.enable = 0 | Sync signals may still be generated (to keep a display "locked") but active video area is blanked |

# 4. All Signals List

## 4.1 External Signals (Chip/Testbench Boundary)

| Signal | Direction (SoC perspective) | Width | Description |
| --- | --- | --- | --- |
| clk | in | 1 | System clock |
| rst_n | in | 1 | Active-low asynchronous system reset |
| heartbeat_in | in | 1 | Heartbeat pulse from simulated main system — direct wire to Heartbeat Monitor |
| reset_out | out | 1 | Active-low reset command to simulated main system — direct wire from Power/Reset Sequencer |
| uart_tx | out | 1 | UART transmit |
| uart_rx | in | 1 | UART receive |
| vga_hsync | out | 1 | VGA horizontal sync (optional 4th IP) |
| vga_vsync | out | 1 | VGA vertical sync (optional 4th IP) |
| vga_rgb | out | 12 | VGA pixel color (optional 4th IP) |
| jtag_tck/tms/tdi/tdo | in/in/in/out | 1 each | Debug Module Interface, core-provided, unmodified |
| gpio_ext[N:0] | in/out (per pin) | N+1 | General-purpose I/O, unused by custom IPs, reserved for any future expansion |

## 4.2 Core ↔ AXI4 Interconnect Signals (core-provided, unmodified)

| Signal Group | Signals | Description |
| --- | --- | --- |
| Write Address Channel | AWADDR, AWLEN, AWSIZE, AWBURST, AWVALID, AWREADY | Write transaction address/control |
| Write Data Channel | WDATA, WSTRB, WLAST, WVALID, WREADY | Write data payload |
| Write Response Channel | BRESP, BVALID, BREADY | Write completion status |
| Read Address Channel | ARADDR, ARLEN, ARSIZE, ARBURST, ARVALID, ARREADY | Read transaction address/control |
| Read Data Channel | RDATA, RRESP, RLAST, RVALID, RREADY | Read data payload |
| Interrupt | irq_i (into core) | External interrupt request line(s), see Section 8 |

## 4.3 AXI-to-Wishbone Bridge Boundary Signals

| Signal | Direction (bridge perspective) | Width | Description |
| --- | --- | --- | --- |
| AXI4 slave side | — | — | Mirrors 3.2 signal set, slave role |
| wb_clk_o | out | 1 | Wishbone clock |
| wb_rst_o | out | 1 | Wishbone reset |
| wb_adr_o | out | 32 | Translated Wishbone address |
| wb_dat_o | out | 32 | Write data to Wishbone bus |
| wb_dat_i | in | 32 | Read data from Wishbone bus |
| wb_we_o | out | 1 | Write enable |
| wb_stb_o | out | 1 | Strobe |
| wb_cyc_o | out | 1 | Cycle valid |
| wb_ack_i | in | 1 | Acknowledge from addressed slave |
| wb_err_i | in | 1 | Error response from addressed slave (see Section 7.6) |

## 4.4 Wishbone Peripheral Bus — Shared Slave Signal Set

Every mandatory peripheral and every custom IP presents this identical interface.

| Signal | Direction (IP perspective) | Width | Description |
| --- | --- | --- | --- |
| wb_clk_i | in | 1 | Bus clock |
| wb_rst_i | in | 1 | Synchronous reset |
| wb_adr_i | in | 8 (local offset) | Register address within this IP |
| wb_dat_i | in | 32 | Write data |
| wb_dat_o | out | 32 | Read data |
| wb_we_i | in | 1 | Write enable |
| wb_stb_i | in | 1 | Strobe |
| wb_cyc_i | in | 1 | Cycle valid |
| wb_ack_o | out | 1 | Acknowledge |
| wb_err_o | out | 1 | Error response (asserted on invalid access, see Section 7.6) |
| wb_sel_i | in | 4 | Byte select |

## 4.5 Point-to-Point Inter-IP Signals (Non-Bus)

| Signal | From → To | Width | Description |
| --- | --- | --- | --- |
| heartbeat_in | Chip pin → Heartbeat Monitor | 1 | Direct wire, bypasses GPIO/Wishbone entirely |
| reset_out | Power/Reset Sequencer → Chip pin | 1 | Direct wire, bypasses GPIO/Wishbone entirely |
| hb_irq | Heartbeat Monitor → Interrupt Controller | 1 | Optional; see Section 8 |
| policy_irq | Recovery Policy → Interrupt Controller | 1 | Optional; see Section 8 |

## 4.6 Internal Signals — Heartbeat Monitor

| Signal | Width | Description |
| --- | --- | --- |
| hb_counter | 32 | Free-running cycle counter since last heartbeat edge |
| hb_threshold_reg | 32 | Latched HB_THRESHOLD |
| hb_unresponsive_flag | 1 | Sticky flag, mirrors HB_STATUS.bit0 |
| hb_state | 2 | FSM state: DISABLED / IDLE / COUNTING / UNRESPONSIVE |
| hb_prev_sample | 1 | Previous-cycle sample of heartbeat_in, for edge detection |
| hb_rising_edge | 1 | Combinational: heartbeat_in & ~hb_prev_sample |

## 4.7 Internal Signals — Power/Reset Sequencer

| Signal | Width | Description |
| --- | --- | --- |
| rst_countdown | 32 | Countdown counter during ASSERT state |
| rst_hold_reg | 32 | Latched RST_HOLD_CYCLES |
| rst_state | 2 | FSM state: IDLE / ASSERT / DEASSERT / DONE |
| rst_in_progress | 1 | Mirrors RST_STATUS.bit0 |
| rst_complete | 1 | Mirrors RST_STATUS.bit1 |

## 4.8 Internal Signals — Recovery Policy / Event Log

| Signal | Width | Description |
| --- | --- | --- |
| log_buffer | 16 × 32 | Circular buffer of event timestamps |
| log_wr_ptr | 4 | Write pointer, wraps at 16 |
| log_count | 32 | Total events logged (saturating) |
| window_counter | 32 | Cycles elapsed in current rolling window |
| window_recovery_count | 8 | Recoveries counted within current window |
| lockout_flag | 1 | Sticky, mirrors POL_STATUS.bit0 |
| policy_window_reg | 32 | Latched POL_WINDOW |
| policy_threshold_reg | 8 | Latched POL_THRESHOLD |

## 4.9 Internal Signals — VGA Controller (optional 4th IP)

| Signal | Width | Description |
| --- | --- | --- |
| pixel_clk | 1 | Divided pixel clock from clk |
| h_count, v_count | 10, 10 | Scan position counters |
| active_video | 1 | High during drawable region |
| dashboard_buffer | implementation-defined | Firmware-written status content |
| vga_refresh_flag | 1 | Set when firmware should refresh buffer |

# 5. Control Path

## 5.1 Path 1 — Automatic Detection → Recovery

1. Testbench drives heartbeat_in, simulating the main system being alive.

2. Heartbeat Monitor counts elapsed cycles since the last pulse, entirely in hardware, every clock cycle, independent of the core.

3. If elapsed count exceeds HB_THRESHOLD, the FSM latches hb_unresponsive_flag — a pure hardware decision.

4. The core discovers this by polling HB_STATUS over Wishbone in its main loop.

5. Before acting, the core reads POL_STATUS from the Recovery Policy IP to check the lockout bit — a deliberate control gate.

6. If not locked out, the core writes RST_CTRL.trigger=1 on the Power/Reset Sequencer.

7. The Reset Sequencer's own FSM (ASSERT → DEASSERT → DONE) runs autonomously once triggered.

8. The core reads the Timer, writes the value to POL_EVENT_TS, then writes POL_CTRL.record_event=1.

9. The Recovery Policy IP evaluates the rolling-window threshold in hardware; sets lockout_flag if exceeded.

10. The core writes HB_CTRL.clear=1, explicitly acknowledging the Heartbeat Monitor — required because the flag does not auto-clear on a resumed heartbeat (see prior design rationale: this guarantees no recovery event is silently missed due to firmware timing).

## 5.2 Path 2 — Manual Override via UART

1. Admin types a command (status, history, force reset, clear lockout).

2. Bytes arrive on uart_rx, buffered in the UART peripheral.

3. Core polls UART over Wishbone, assembles a line buffer.

4. Command parser dispatches based on the parsed string.

5. force reset and clear lockout reuse steps 6–9 and the lockout-clear write path from 4.1 respectively.

6. Response assembled in firmware, written out via UART transmit path.

## 5.3 Path 3 — VGA Dashboard Refresh

1. Each main-loop iteration (or Timer-paced interval), the core reads current status from Heartbeat Monitor and Recovery Policy IPs.

2. Firmware writes formatted content into the VGA Controller's dashboard buffer over Wishbone.

3. The VGA Controller's pixel-generation logic runs independently thereafter, continuously rendering whatever is in the buffer.

## 5.4 Control Path Summary Table

| Trigger | Initiator | Path | Result |
| --- | --- | --- | --- |
| Heartbeat timeout | Hardware (Heartbeat Monitor) | Monitor → Core (poll) → Reset Sequencer → Recovery Policy | Automatic recovery + log, or blocked if locked out |
| force reset command | Firmware (UART) | Core → Reset Sequencer → Recovery Policy | Manual recovery, always executes |
| clear lockout command | Firmware (UART) | Core → Recovery Policy | Lockout flag manually cleared |
| Dashboard refresh | Firmware (main loop) | Core → (reads) → VGA Controller (write) | Live visual update |

# 6. Data Path

## 6.1 Data Flow 1 — Heartbeat Timing Data

heartbeat_in → Heartbeat Monitor's counter (continuous, hardware) → HB_ELAPSED/HB_STATUS → core (Wishbone read) → UART/VGA display.

## 6.2 Data Flow 2 — Timestamp Data

Timer's free-running count → core (read) → POL_EVENT_TS (write) → log_buffer entry (on trigger). One-shot, per recovery event.

## 6.3 Data Flow 3 — Event Log Readback Data

Core writes LOG_READ_IDX → Recovery Policy read mux selects log_buffer slot → LOG_READ_DATA → core reads → UART history output or VGA recent-entries display.

## 6.4 Data Flow 4 — Recovery Count / Window Data

window_counter/window_recovery_count (internal, Recovery Policy) → POL_STATUS → core reads → used as (a) control-path decision input, (b) UART/VGA display data.

## 6.5 Data Flow 5 — Reset Pulse Data

Core writes RST_HOLD_CYCLES (typically once, at boot) → Reset Sequencer's countdown register → on trigger, drives reset_out low for that duration → exits chip directly to testbench.

## 6.6 Data Flow 6 — UART Command/Response Text

uart_rx bits → UART deserializes to bytes → core reads (Wishbone) → assembled command string → parsed → triggers reads from other flows → core formats response → written back to UART (Wishbone) → serialized on uart_tx.

## 6.7 Data Flow 7 — VGA Pixel Data

Aggregated data from flows 1, 3, 4 → written to VGA dashboard buffer (Wishbone) → read continuously by pixel-logic (decoupled from core) → combined with scan position → rgb/hsync/vsync output.

## 6.8 Data Path Summary Table

| Data | Produced By | Consumed By | Transport |
| --- | --- | --- | --- |
| Heartbeat elapsed time | Heartbeat Monitor | Core, then UART/VGA | Wishbone read |
| Unresponsive flag | Heartbeat Monitor | Core (decision) | Wishbone read |
| Timestamp | Timer | Core → Recovery Policy | Wishbone read then write |
| Event log entries | Recovery Policy | Core → UART/VGA | Wishbone read |
| Lockout flag / count | Recovery Policy | Core (decision), UART/VGA | Wishbone read |
| Reset hold duration | Core (config) | Power/Reset Sequencer | Wishbone write |
| Reset pulse | Power/Reset Sequencer | GPIO pin → simulated main system | Direct wire |
| UART command text | External terminal | Core (parser) | Wishbone read |
| UART response text | Core | External terminal | Wishbone write |
| Dashboard content | Core (aggregated) | VGA Controller → display | Wishbone write |

# 7. Error Cases and Non-Fatal Recovery Paths

Every error case below is designed so the system degrades gracefully and continues operating — no error condition in this design should be allowed to halt the SoC or require a full system reset to recover from, since a management controller that itself needs rescuing defeats its purpose.

## 7.1 Heartbeat Monitor Error Cases

Error 7.1.1 — `HB_THRESHOLD` configured to 0

- Condition: Firmware writes HB_THRESHOLD = 0, meaning any single missed cycle would trigger unresponsive.

- Effect if unhandled: Monitor would flag unresponsive almost immediately after enable, even under normal operation — a false-positive storm.

- Recovery path (non-fatal): IP enforces a minimum threshold floor in hardware (e.g., reject writes below a defined HB_THRESHOLD_MIN, silently clamping to the minimum rather than accepting 0). Firmware can additionally sanity-check the value it intends to write before writing it. System continues operating with the clamped, safe threshold — no crash, no lockup.

Error 7.1.2 — Spurious/glitch pulses on `heartbeat_in`

- Condition: Electrical noise or a testbench timing error produces a very short, spurious pulse that isn't a real heartbeat.

- Effect if unhandled: Could cause hb_counter to reset prematurely, incorrectly masking a real fault (system looks "alive" when it isn't).

- Recovery path (non-fatal): Add a minimum pulse-width qualifier (simple debounce counter) before accepting an edge as a valid heartbeat reset. A pulse shorter than the qualifier duration is ignored; the counter is unaffected and continues toward threshold normally. No error state entered — this is filtered silently and system continues counting correctly.

Error 7.1.3 — Firmware never clears a latched unresponsive flag

- Condition: Firmware bug or crash means HB_CTRL.clear is never written after a recovery.

- Effect if unhandled: Monitor stays latched in UNRESPONSIVE state indefinitely; HB_ELAPSED keeps growing but the monitor cannot re-detect a *second* failure cleanly, since it's already latched.

- Recovery path (non-fatal): This is a legitimate observable condition, not a hidden failure — HB_STATUS remains readable at all times, so a UART status query or the VGA dashboard will show UNRESPONSIVE persisting, visibly signaling to an admin that firmware itself may need attention. The hardware itself does not crash or hang; it simply reports the state accurately and waits. This is analogous to the intentional lockout behavior — visibility over silent failure.

Error 7.1.4 — `hb_counter` overflow

- Condition: Monitor left enabled with no incoming pulses for longer than 2^32 cycles.

- Effect if unhandled: Counter wraps to 0, HB_ELAPSED would misleadingly read a small value despite the system having been unresponsive for a very long time.

- Recovery path (non-fatal): Counter saturates at 32'hFFFF_FFFF instead of wrapping (saturating-add logic instead of free wraparound). HB_ELAPSED correctly reads "maximum representable time," never rolls back to a misleadingly small number. Purely a counter-logic choice; no functional failure.

## 7.2 Power/Reset Sequencer Error Cases

Error 7.2.1 — Re-trigger received while a sequence is already in progress

- Condition: Firmware (or a race between automatic and manual trigger paths) writes RST_CTRL.trigger=1 while rst_state != IDLE.

- Effect if unhandled (if not explicitly defined): Could restart the countdown mid-pulse, producing an unpredictable, non-conforming reset pulse width.

- Recovery path (non-fatal): Explicitly defined and documented behavior (Section 9.2): the trigger is ignored while a sequence is in progress. The current sequence completes normally, uninterrupted. RST_STATUS.bit0 remaining high is itself the signal to firmware that a second trigger would be redundant; firmware can poll before re-triggering if it needs a subsequent guaranteed-fresh sequence.

Error 7.2.2 — `RST_HOLD_CYCLES` configured to 0

- Condition: Firmware writes 0 to the hold-duration register.

- Effect if unhandled: reset_out would pulse for a single cycle or less — potentially too short to be a valid reset for the simulated main system.

- Recovery path (non-fatal): IP enforces a minimum hold value (RST_HOLD_MIN), clamping any write below it. Sequence always produces at least a minimum-valid-width pulse. No fatal condition; system continues.

Error 7.2.3 — `reset_out` observed stuck low (simulated stuck-at fault, for verification purposes)

- Condition: A deliberately injected fault (in verification only) where reset_out fails to return high after DEASSERT.

- Effect if unhandled: Main system would be held in permanent reset.

- Recovery path (non-fatal, verification-only): Not a hardware self-healing case — this is a test scenario to prove your FSM's DEASSERT → DONE transition is correct and doesn't have a bug allowing this. Documented as a verification test case (see companion Verification Plan) rather than a runtime-recoverable error, since a true stuck-at-fault is a physical hardware defect outside what firmware/hardware logic can work around.

## 7.3 Recovery Policy / Event Log Error Cases

Error 7.3.1 — Log buffer full (more than 16 events)

- Condition: More than LOG_DEPTH (16) events recorded since last reset.

- Effect if unhandled: Could silently drop new events or corrupt old ones.

- Recovery path (non-fatal): Circular buffer design (documented simplification, Section 9.3) — oldest entry is deliberately overwritten by the newest. LOG_COUNT saturates at 16 rather than wrapping, so firmware/UART can distinguish "exactly 16 logged" from "more than 16 have occurred, oldest were overwritten" if an additional overflow-indicator bit is added (recommended enhancement: add LOG_OVERFLOW sticky bit to POL_STATUS to make this explicit rather than silent).

Error 7.3.2 — `POL_WINDOW` or `POL_THRESHOLD` configured to 0

- Condition: Firmware writes 0 to either.

- Effect if unhandled: POL_WINDOW=0 could cause the window to reset every cycle, effectively disabling lockout entirely (never accumulates count); POL_THRESHOLD=0 could cause immediate lockout on the very first event, defeating the "allow some retries" purpose.

- Recovery path (non-fatal): Both registers enforce a minimum floor (POL_WINDOW_MIN, POL_THRESHOLD_MIN >= 1), clamping writes below the minimum. System continues with safe defaults; no crash or undefined behavior.

Error 7.3.3 — Simultaneous automatic and manual event recording (race condition)

- Condition: An automatic recovery (Path 4.1) and a manual force reset (Path 4.2) are triggered in close succession, potentially both attempting to write POL_CTRL.record_event near-simultaneously.

- Effect if unhandled: Wishbone bus is inherently single-master-per-transaction from the core's perspective (Section on master/slave roles — there is only one master, the core), so true simultaneous writes cannot occur at the bus level; however, firmware logic itself could issue two closely-spaced writes without waiting for the first to be fully processed.

- Recovery path (non-fatal): IP processes POL_CTRL writes sequentially as they arrive on the single-master bus — no special hardware handling needed since the bus protocol itself serializes all transactions. Firmware is designed (Section 10) to wait for Wishbone ack before issuing the next write, which naturally prevents any overlap. This is inherently safe by construction, not a case requiring extra recovery logic — documented here to show the race was considered and is structurally precluded.

Error 7.3.4 — Lockout never clears (stuck locked-out)

- Condition: Admin never issues clear lockout, or firmware bug prevents the command from reaching the IP.

- Effect if unhandled: System would remain in LOCKED OUT state indefinitely — but this is the *intended* fail-safe behavior, not a bug, since the entire point is that automatic recovery should not resume without explicit human decision.

- Recovery path (non-fatal, by design): State is fully visible via POL_STATUS, UART status, and VGA dashboard at all times — an admin can always see and manually clear it. This is a deliberate fail-safe, not an error requiring hardware-side recovery.

## 7.4 UART / Console Error Cases

Error 7.4.1 — Malformed or unrecognized command

- Condition: Admin sends a command string that doesn't match any known command.

- Recovery path (non-fatal): Parser falls through to a default case, prints "Unknown command" (see reference console.c pseudocode, Section 10.4), and returns to waiting for the next line. No state changes, no crash.

Error 7.4.2 — Command buffer overflow

- Condition: Admin sends a line longer than the firmware's fixed buffer size (e.g. 64 bytes) without a newline.

- Effect if unhandled: Potential buffer overrun, memory corruption.

- Recovery path (non-fatal): Firmware enforces a hard cap on bytes accepted per line; once the cap is reached without a newline, the partial buffer is discarded and an error message ("Command too long, discarded") is returned. Parser resumes listening for a fresh line.

## 7.5 Bus / Address Decode Error Cases

Error 7.5.1 — Access to an unmapped address

- Condition: Firmware (due to a bug) issues a Wishbone read/write to an address not claimed by any peripheral.

- Effect if unhandled: Bus could hang waiting for an ack that never arrives (classic bus lockup).

- Recovery path (non-fatal): The Wishbone decoder/bridge implements a timeout counter — if no slave asserts wb_ack_o within a bounded number of cycles, the bridge asserts wb_err_o back to the core instead of hanging indefinitely. Firmware's driver-layer read/write wrapper functions check for this error response and can log/report it via UART rather than the whole system hanging.

Error 7.5.2 — Write to a read-only register (e.g., `HB_STATUS`, `HB_ELAPSED`)

- Condition: Firmware bug attempts to write to a register documented as read-only.

- Recovery path (non-fatal): IP's Wishbone write-handling logic simply ignores writes to read-only offsets (falls into the default: ; case, as implemented in the Heartbeat Monitor RTL) — the write is silently accepted at the bus level (still asserts ack, so firmware doesn't hang) but has no effect. No corruption, no error propagation needed for this class of mistake.

## 7.6 VGA Controller Error Cases (optional 4th IP)

Error 7.6.1 — Dashboard buffer not updated (stale display)

- Condition: Firmware fails to refresh the dashboard buffer for an extended period (e.g., stuck in a long operation).

- Effect if unhandled: Display shows stale data without indication it's stale.

- Recovery path (non-fatal): Recommended enhancement — VGA Controller tracks cycles since last buffer write internally and can render a subtle "stale" indicator (e.g., a small icon or dimmed text) if the buffer hasn't been touched within an expected refresh interval. Purely cosmetic safeguard; does not affect UART-based status accuracy, which remains independently correct.

# 8. All Possible Interrupts

The baseline design (as specified in Sections 4 and 9) uses firmware polling, not interrupts, for simplicity and reduced verification risk given project time constraints (this trade-off was discussed and deliberately chosen). This section documents the full set of interrupt sources that could be wired into VeeR EL2's interrupt controller (PIC) as an enhancement, so the interrupt architecture is fully specified even though the default configuration leaves them unused/masked.

## 8.1 Interrupt Source Table

| Interrupt | Source IP | Trigger Condition | Priority (suggested) | Default State |
| --- | --- | --- | --- | --- |
| hb_irq | Heartbeat Monitor | hb_unresponsive_flag transitions 0→1 | High | Masked (polling used instead) |
| policy_irq | Recovery Policy | lockout_flag transitions 0→1 | Highest | Masked (polling used instead) |
| rst_done_irq | Power/Reset Sequencer | FSM reaches DONE state | Medium | Masked (polling used instead) |
| uart_rx_irq | UART (mandatory, core-provided) | Receive FIFO non-empty | Medium | Masked (polling used instead) — may already be available as a standard feature of the provided UART core |
| wb_err_irq | AXI-to-Wishbone Bridge | wb_err_o asserted (see Error 7.5.1) | High | Recommended to remain enabled even in the polling baseline, since a bus timeout is a genuine anomaly firmware should be notified of immediately rather than discovering via a stalled read |

## 8.2 Interrupt Priority Rationale

- `policy_irq` (highest) — a newly-set lockout is the most operationally significant event in the system (automatic recovery has stopped); prompt firmware awareness matters most here if interrupts are enabled.

- `hb_irq` (high) — a new unresponsive detection is time-sensitive since it starts the recovery clock.

- `wb_err_irq` (high) — a bus error indicates a firmware/addressing bug; should not be silently ignored even in an otherwise polled design.

- `rst_done_irq` (medium) — useful for firmware to avoid polling RST_STATUS in a tight loop while a sequence completes, but not time-critical.

- `uart_rx_irq` (medium) — standard console responsiveness improvement, not correctness-critical for this design's core behavior.

## 8.3 Interrupt Handling Model (if enabled as a future enhancement)

If enabled, each interrupt would route through VeeR EL2's Programmable Interrupt Controller (PIC) as an external interrupt source, each with its own vector. The corresponding ISR would perform the minimum necessary work (read the triggering status register, clear/acknowledge at the source, set a flag for the main loop to act on) and return quickly — consistent with standard embedded interrupt-handling practice of keeping ISRs short and deferring larger processing (recovery sequencing, logging, UART output) to the main loop, which would check the ISR-set flags on its next iteration.

## 8.4 Why the Baseline Uses Polling Instead

As established earlier in the design process: this application has no genuine microsecond-level latency requirement — the reliability story (detect, recover, escalate) is unaffected by whether the core notices a flag one loop iteration sooner or later, given loop iterations execute far faster than any of the configured thresholds/windows. Implementing interrupts correctly requires configuring VeeR EL2's PIC, writing and verifying ISRs, and managing interrupt masking/priority — meaningful additional implementation and verification risk for a benefit that doesn't materially change system behavior here. Interrupts remain fully specified in this section as a documented, well-understood upgrade path, not a design gap.

# 9. Register Descriptions

## 9.1 Heartbeat Monitor (base `0x1001_0000`)

| Offset | Name | Width | R/W | Reset Value | Bit Fields |
| --- | --- | --- | --- | --- | --- |
| 0x00 | HB_CTRL | 32 | R/W | 0x0 | [0] enable — 1=monitor active. [1] clear — write 1 to acknowledge/clear unresponsive flag (self-clearing, only effective while in UNRESPONSIVE state). [31:2] reserved, read as 0. |
| 0x04 | HB_THRESHOLD | 32 | R/W | 0x000F4240 | [31:0] maximum cycles allowed between heartbeat pulses before flagging unresponsive. Clamped to HB_THRESHOLD_MIN if a lower value is written (Error 7.1.1). |
| 0x08 | HB_STATUS | 32 | R | 0x0 | [0] unresponsive — sticky, 1=system flagged unresponsive. [1] heartbeat_in current live level. [31:2] reserved, read as 0. |
| 0x0C | HB_ELAPSED | 32 | R | 0x0 | [31:0] live count of cycles since last heartbeat rising edge; saturates rather than wraps (Error 7.1.4). |

## 9.2 Power/Reset Sequencer (base `0x1001_0100`)

| Offset | Name | Width | R/W | Reset Value | Bit Fields |
| --- | --- | --- | --- | --- | --- |
| 0x00 | RST_CTRL | 32 | W | 0x0 | [0] trigger — write 1 to start reset sequence; ignored if a sequence is already in progress (Error 7.2.1). [31:1] reserved. |
| 0x04 | RST_HOLD_CYCLES | 32 | R/W | 0x00002710 | [31:0] number of cycles reset_out is held asserted (low). Clamped to RST_HOLD_MIN if a lower value is written (Error 7.2.2). |
| 0x08 | RST_STATUS | 32 | R | 0x0 | [0] in-progress — 1 while sequence is running. [1] complete — sticky, set on DONE, cleared on next trigger. [31:2] reserved. |

## 9.3 Recovery Policy / Event Log (base `0x1001_0200`)

| Offset | Name | Width | R/W | Reset Value | Bit Fields |
| --- | --- | --- | --- | --- | --- |
| 0x00 | POL_CTRL | 32 | W | 0x0 | [0] record_event — write 1 to log the value currently staged in POL_EVENT_TS. [1] clear_lockout — write 1 to manually clear the lockout flag. [31:2] reserved. |
| 0x04 | POL_EVENT_TS | 32 | W | 0x0 | [31:0] timestamp to associate with the next recorded event; firmware writes this immediately before writing POL_CTRL.record_event. |
| 0x08 | POL_WINDOW | 32 | R/W | 0x000F4240 | [31:0] rolling window length in cycles. Clamped to POL_WINDOW_MIN (Error 7.3.2). |
| 0x0C | POL_THRESHOLD | 32 | R/W | 0x3 | [7:0] max recoveries allowed within a window before lockout. Clamped to minimum of 1 (Error 7.3.2). [31:8] reserved. |
| 0x10 | POL_STATUS | 32 | R | 0x0 | [0] locked_out — sticky. [15:8] current recovery count in active window. [16] LOG_OVERFLOW (recommended addition, Error 7.3.1) — sticky, set if more than LOG_DEPTH events have occurred since last reset. [31:17] reserved. |
| 0x14 | LOG_READ_IDX | 32 | R/W | 0x0 | [3:0] index (0–15) selecting which log entry LOG_READ_DATA returns. [31:4] reserved. |
| 0x18 | LOG_READ_DATA | 32 | R | 0x0 | [31:0] timestamp stored at LOG_READ_IDX. |
| 0x1C | LOG_COUNT | 32 | R | 0x0 | [31:0] total events logged since reset; saturates at LOG_DEPTH (16). |

## 9.4 VGA Controller (base `0x1001_0300`, optional 4th IP)

| Offset | Name | Width | R/W | Reset Value | Bit Fields |
| --- | --- | --- | --- | --- | --- |
| 0x00 | VGA_CTRL | 32 | R/W | 0x0 | [0] dashboard enable. [31:1] reserved. |
| 0x04 | VGA_REFRESH_DIV | 32 | R/W | implementation-default | [31:0] optional pacing divider for how often firmware-pushed content is expected to update relative to pixel clock. |
| 0x08+ | VGA_BUFFER[...] | implementation-defined | W | — | Text/graphics content buffer; exact layout depends on selected base VGA core (character-cell vs. bitmap) — see Open Items. |

## 9.5 Mandatory Peripheral Registers

Not re-specified here — UART, GPIO, and Timer registers follow their respective provided core's existing documented register maps (from the VeeR reference SoC / chosen open-source cores). Firmware driver wrappers (Section 10.3) abstract these behind simple function calls regardless of exact underlying offsets.

# 10. Application of the Project — The C Application in Detail

## 10.1 Real-World Application Recap

This SoC is a scaled-down, verifiable model of the reliability layer real datacenters depend on: unattended recovery from software hangs, protection against restart-loops making a hardware fault worse, remote visibility without physical access, timestamped post-incident diagnosis, and live operational monitoring — all implemented as a working, testable system rather than an abstract description.

## 10.2 Firmware Design Goals

- Keep all time-critical logic in hardware; firmware is purely reactive/administrative.

- Simple superloop architecture — no RTOS, no task scheduler, minimizing implementation and verification complexity given project time constraints.

- Clear separation between low-level register access (drivers) and application logic (main loop, console).

- Every register access wrapped in a named function — no raw pointer dereferences scattered through application code, for readability and easier debugging/defense.

## 10.3 Module Structure

main.c - init, superloop drivers/ heartbeat.c/.h - HB_* register wrappers reset_seq.c/.h - RST_* register wrappers policy.c/.h - POL_*/LOG_* register wrappers uart.c/.h - line-based UART I/O vga.c/.h - dashboard buffer writes timer.c/.h - Timer read/config console.c/.h - command parser

### 9.3.1 `drivers/heartbeat.h` — interface

#ifndef HEARTBEAT_H #define HEARTBEAT_H #include <stdint.h> #include <stdbool.h> void heartbeat_init(uint32_t threshold_cycles); bool heartbeat_is_unresponsive(void); uint32_t heartbeat_elapsed(void); void heartbeat_clear_flag(void); #endif

### 9.3.2 `drivers/heartbeat.c` — implementation (register-level detail)

#include "heartbeat.h" #define HB_BASE 0x10010000u #define HB_CTRL (*(volatile uint32_t*)(HB_BASE + 0x00)) #define HB_THRESHOLD (*(volatile uint32_t*)(HB_BASE + 0x04)) #define HB_STATUS (*(volatile uint32_t*)(HB_BASE + 0x08)) #define HB_ELAPSED (*(volatile uint32_t*)(HB_BASE + 0x0C)) void heartbeat_init(uint32_t threshold_cycles) { HB_THRESHOLD = threshold_cycles; HB_CTRL = 0x1; // enable = 1, clear = 0 } bool heartbeat_is_unresponsive(void) { return (HB_STATUS & 0x1) != 0; } uint32_t heartbeat_elapsed(void) { return HB_ELAPSED; } void heartbeat_clear_flag(void) { HB_CTRL = 0x3; // enable = 1, clear = 1 (self-clearing on hardware side) }

### 9.3.3 `drivers/reset_seq.h` / `.c` — interface and key logic

// reset_seq.h void heartbeat_and_reset_wait(void); // helper, see below void reset_seq_init(uint32_t hold_cycles); void reset_seq_trigger(void); bool reset_seq_is_complete(void);

// reset_seq.c (excerpt) #define RST_BASE 0x10010100u #define RST_CTRL (*(volatile uint32_t*)(RST_BASE + 0x00)) #define RST_HOLD (*(volatile uint32_t*)(RST_BASE + 0x04)) #define RST_STATUS (*(volatile uint32_t*)(RST_BASE + 0x08)) void reset_seq_init(uint32_t hold_cycles) { RST_HOLD = hold_cycles; } void reset_seq_trigger(void) { RST_CTRL = 0x1; } bool reset_seq_is_complete(void) { return (RST_STATUS & 0x2) != 0; }

### 9.3.4 `drivers/policy.h` / `.c` — interface and key logic

// policy.h void policy_init(uint32_t window_cycles, uint8_t threshold); bool policy_is_locked_out(void); uint8_t policy_window_count(void); void policy_record_event(uint32_t timestamp); void policy_clear_lockout(void); uint32_t policy_log_count(void); uint32_t policy_log_read(uint8_t idx);

// policy.c (excerpt) #define POL_BASE 0x10010200u #define POL_CTRL (*(volatile uint32_t*)(POL_BASE + 0x00)) #define POL_EVENT_TS (*(volatile uint32_t*)(POL_BASE + 0x04)) #define POL_WINDOW (*(volatile uint32_t*)(POL_BASE + 0x08)) #define POL_THRESHOLD (*(volatile uint32_t*)(POL_BASE + 0x0C)) #define POL_STATUS (*(volatile uint32_t*)(POL_BASE + 0x10)) #define LOG_READ_IDX (*(volatile uint32_t*)(POL_BASE + 0x14)) #define LOG_READ_DATA (*(volatile uint32_t*)(POL_BASE + 0x18)) #define LOG_COUNT_REG (*(volatile uint32_t*)(POL_BASE + 0x1C)) void policy_record_event(uint32_t timestamp) { POL_EVENT_TS = timestamp; POL_CTRL = 0x1; // record_event } bool policy_is_locked_out(void) { return (POL_STATUS & 0x1) != 0; } void policy_clear_lockout(void) { POL_CTRL = 0x2; // clear_lockout } uint32_t policy_log_read(uint8_t idx) { LOG_READ_IDX = idx; return LOG_READ_DATA; }

## 10.4 Main Application Logic

### 9.4.1 Initialization sequence

int main(void) { heartbeat_init(HB_THRESHOLD_DEFAULT); reset_seq_init(RST_HOLD_DEFAULT); policy_init(POL_WINDOW_DEFAULT, POL_THRESHOLD_DEFAULT); uart_init(UART_BAUD_115200); vga_init(); uart_print("BMC firmware initialized. Monitoring active.\n"); main_loop(); return 0; // never reached }

### 9.4.2 Main superloop

void main_loop(void) { while (1) { // 1. Automatic detection/recovery path (Control Path 4.1) if (heartbeat_is_unresponsive()) { if (!policy_is_locked_out()) { reset_seq_trigger(); while (!reset_seq_is_complete()) { /* wait */ } policy_record_event(timer_now()); heartbeat_clear_flag(); uart_print("Recovery triggered and logged.\n"); } else { uart_print("LOCKOUT ACTIVE: manual attention required.\n"); } } // 2. Console command servicing (Control Path 4.2) if (uart_has_line()) { char buf[64]; uart_read_line(buf, sizeof(buf)); console_handle_command(buf); } // 3. VGA dashboard refresh (Control Path 4.3) vga_update_dashboard( heartbeat_is_unresponsive(), heartbeat_elapsed(), policy_is_locked_out(), policy_window_count(), policy_log_count() ); } }

### 9.4.3 Console command parser

void console_handle_command(const char *cmd) { if (strcmp(cmd, "status") == 0) { uart_printf("System: %s\n", heartbeat_is_unresponsive() ? "UNRESPONSIVE" : "ONLINE"); uart_printf("Lockout: %s\n", policy_is_locked_out() ? "YES" : "NO"); uart_printf("Time since heartbeat: %lu cycles\n", heartbeat_elapsed()); uart_printf("Recoveries this window: %u\n", policy_window_count()); } else if (strcmp(cmd, "history") == 0) { uint32_t n = policy_log_count(); for (uint32_t i = 0; i < n; i++) { uart_printf("Event %lu: t=%lu\n", i, policy_log_read((uint8_t)i)); } } else if (strcmp(cmd, "force reset") == 0) { reset_seq_trigger(); while (!reset_seq_is_complete()) { /* wait */ } policy_record_event(timer_now()); uart_print("Manual reset triggered and logged.\n"); } else if (strcmp(cmd, "clear lockout") == 0) { policy_clear_lockout(); uart_print("Lockout cleared. Automatic recovery resumed.\n"); } else if (strncmp(cmd, "", 64) == 0) { // empty line, ignore silently } else { uart_print("Unknown command. Try: status ¦ history ¦ force reset ¦ clear lockout\n"); } }

## 10.5 Error Handling in Firmware

Firmware-level error handling ties directly to Section 7:

- Every register-write wrapper (drivers) is a simple, direct store — no bounds-checking needed in firmware itself for out-of-range configuration values, since the hardware IPs enforce minimum floors (Errors 6.1.1, 6.2.2, 6.3.2) at the register-write level. This is a deliberate design choice: correctness lives in hardware, firmware stays simple.

- Bus-level errors (Error 7.5.1) are handled by checking a bounded retry/timeout in the driver wrappers, rather than assuming every Wishbone transaction succeeds instantly — a production-quality version of the drivers shown above would wrap each access with a timeout check against wb_err_o, omitted here for clarity but noted as a required hardening step before final submission.

- UART command errors (Error 7.4.1, 6.4.2) are handled entirely within console_handle_command() and uart_read_line() as shown.

## 10.6 Build & Toolchain Notes

- Compile with the RISC-V GCC toolchain (riscv32-unknown-elf-gcc), targeting RV32IMC to match VeeR EL2.

- Firmware linked to place code at the instruction memory base and initialized data at the data memory base, per the linker script matching the register base addresses in Section 9 (finalized once actual reference SoC addresses are confirmed, per Open Items).

- Output .hex/.mem file used to preload instruction memory in RTL simulation (via $readmemh or equivalent), consistent with the project's simulation plan.

## 10.7 Summary — What the C Application Demonstrates

The firmware is intentionally simple: initialization, a polling superloop, and a small command parser. Its simplicity is itself a design statement, matching the project's core architectural principle — nearly all meaningful correctness and timing guarantees live in the custom hardware IPs (Sections 6 and 8), and the C application exists only to sequence, configure, and report, never to perform or guarantee the reliability behavior itself.